| _______________________ Automotive Workshop Front & rear brake inspection with measurements, pads, discs, drums, hydraulics, road test | Phone: ____________________ Email: ____________________ Address: __________________ |
| --- | --- |

BRAKE INSPECTION CHECKLIST

| YEAR | MAKE | MODEL |
| --- | --- | --- |
| VIN | REGO / STATE | ODOMETER (KM) |
| TECHNICIAN | DATE (DD/MM/YYYY) | JOB NO. |

■ Pass — within spec    ■ Needs Attention — monitor / service soon    ■ Fail — safety concern / repair now

Record pad thickness in mm. Many manufacturers call for replacement at 3 mm or less, check the spec for the vehicle.

| Inspection Item | Pass | Needs Attn | Fail | Measure | Notes / Measurements |
| --- | --- | --- | --- | --- | --- |
| FRONT BRAKES |  |  |  |  |  |
| Pad thickness, LF inner / outer (mm) | ☐ | ☐ | ☐ | ☐ |  |
| Pad thickness, RF inner / outer (mm) | ☐ | ☐ | ☐ | ☐ |  |
| Disc thickness, LF (mm) vs min spec | ☐ | ☐ | ☐ | ☐ |  |
| Disc thickness, RF (mm) vs min spec | ☐ | ☐ | ☐ | ☐ |  |
| Disc runout / scoring / heat spots | ☐ | ☐ | ☐ | ☐ |  |
| Calipers, leaks, slide pins, boots | ☐ | ☐ | ☐ | ☐ |  |
| Pad hardware & anti-rattle clips | ☐ | ☐ | ☐ | ☐ |  |
| REAR BRAKES |  |  |  |  |  |
| Pad / shoe thickness, LR (mm) | ☐ | ☐ | ☐ | ☐ |  |
| Pad / shoe thickness, RR (mm) | ☐ | ☐ | ☐ | ☐ |  |
| Disc thickness or drum diameter, LR | ☐ | ☐ | ☐ | ☐ |  |
| Disc thickness or drum diameter, RR | ☐ | ☐ | ☐ | ☐ |  |
| Wheel cylinders / calipers, leaks | ☐ | ☐ | ☐ | ☐ |  |
| Springs, adjusters & hardware | ☐ | ☐ | ☐ | ☐ |  |
| Handbrake shoes (if separate) | ☐ | ☐ | ☐ | ☐ |  |
| HYDRAULICS |  |  |  |  |  |
| Brake fluid, level | ☐ | ☐ | ☐ | ☐ |  |
| Brake fluid, condition / moisture % (test strip) | ☐ | ☐ | ☐ | ☐ |  |
| Master cylinder, leaks, cap seal | ☐ | ☐ | ☐ | ☐ |  |
| Brake lines, corrosion, damage | ☐ | ☐ | ☐ | ☐ |  |
| Flexible hoses, cracks, bulges, chafing | ☐ | ☐ | ☐ | ☐ |  |
| ABS module & wheel speed sensors | ☐ | ☐ | ☐ | ☐ |  |
| PEDAL & HANDBRAKE |  |  |  |  |  |
| Pedal height & free play | ☐ | ☐ | ☐ | ☐ |  |
| Pedal feel, firm, no sinking | ☐ | ☐ | ☐ | ☐ |  |
| Brake booster, vacuum / hydraulic assist | ☐ | ☐ | ☐ | ☐ |  |
| Handbrake, travel & holds on incline | ☐ | ☐ | ☐ | ☐ |  |
| Brake / ABS warning lights | ☐ | ☐ | ☐ | ☐ |  |
| ROAD TEST |  |  |  |  |  |
| Stops straight, no pull left / right | ☐ | ☐ | ☐ | ☐ |  |
| No pulsation through pedal or wheel | ☐ | ☐ | ☐ | ☐ |  |
| No grinding, squeal or noise | ☐ | ☐ | ☐ | ☐ |  |
| ABS engages on hard stop (safe area) | ☐ | ☐ | ☐ | ☐ |  |

MEASUREMENTS, SPECS USED, AND RECOMMENDED SERVICE

Technician signature: ______________________          Customer signature: ______________________